PROPOSAL for ONR

“e-Zapper”

A Portable High-Energy Atmospheric Electron Beam Device to

Remotely Deactivate Electronic Devices.

James F. Ziegler

Martin E. Nelson

United States Naval Academy

PROPOSAL for ONR

“e-Zapper”

A Portable High-Energy Atmospheric Electron Beam Device to

Remotely Deactivate Electronic Devices.

Principal Investigators:

James F. Ziegler	Martin Nelson

Professor (v) of Electrical Engineering	Professor of Nuclear Engineering

352 Maury Hall, M.S. 14B	Rickover Hall

U. S. Naval Academy	U. W. Naval Academy

Annapolis, MD,  21402	Annapolis, MD, 21402

410-798-7747	410-293-6427

Ziegler@USNA.edu	Nelson@USNA.edu

Participating Scientists:

Harold Hughes	Kenneth Purser

Manager of Solid State Devices, Code 6816	Southern Cross Co.

Naval Research Laboratories	360 N. Emerson Rd.

4555 Overlook Ave., SW	Lexington, MA 02420

Washington, D.C., 20375	781-956-6613

202-767-2429	KPurser@shore.net

Hughes@estd.nrl.navy.mil

Philip Miller	James D. Zuber

Dir. Div. of Dynamics and Diagnostics, Code 920	Asst. Professor of Electrical Engineering

Research & Technology Department	228 Maury Hall, M.S. 14B

Naval Surface Warfare Center-Indian Head	U. S. Naval Academy

Indian Head, MD, 20640	Annapolis, MD,  21402

301-744-4106	410-293-6164

MoillerPJ@IH.navy.mil	Zuber@USNA.edu

DATES OF ENTIRE WORK UNIT PERIOD: 1 Oct., 2004 – 30 Sept., 2008

USE OF HUMAN SUBJECT : 	none

USE OF ANIMALS:	none

INVESTIGATIONAL DRUGS, DEVICES, BIOLOGICS	none

PERSONNEL HAZARDS	Radiation, Explosives

ENVIRONMENTAL HAZARDS	none

PROPOSED BUDGET:

|  | Direct Costs | Indirect Costs | Total Costs |
| --- | --- | --- | --- |
| Phase 1 | $ 140,000 | $ 0 | $ 140,000 |
| Phase 2 | $ 387,000 | $ 0 | $ 387,000 |
| Phase 3 | $ 363,500 | $ 0 | $ 363,500 |
| Total Costs | $ 890,500 | $ 0 | $ 890,500 |

I agree to accept responsibility for the scientific conduct of the project and to provide the required progress reports and agreed upon products to the OFFICE OF NAVAL RESEARCH.

SIGNATURE AND DATES:

________________________________________	______________________________________

James F. Ziegler, Principal Investigator   DATE	Martin Nelson, Principle Investigator    DATE

Table of Contents

EXECUTIVE SUMMARY	4

Overview:	4

Background	4

Device Construction	4

Experimental Background:	5

TECHNICAL SECTION	6

(1) Scientific Technical Merit	6

(2) Understanding of Navy / EOD Requirements	6

(3) Potential for Field Transition	6

INTRODUCTION	7

PROJECT TIMELINE	9

Phase I - Experimental Evaluation and Prototype Development (12 months)	9

Phase II - Prototype Construction and Testing (18 months)	9

Phase III - Prototype Evaluation and Field Trials (18 months)	10

MANAGEMENT SECTION	12

(4) USNA/NRL Capability, Experience, Facilities and Techniques	12

(5) Qualifications of Key Team Personnel	12

COST SECTION	13

(6) Summary of Proposed Costs and Fees	13

Attachment A – CV of James Ziegler	15

Attachment B – CV of Martin Nelson	31

EXECUTIVE SUMMARY

#### “e- Zapper”

A Portable High-Energy Atmospheric Electron Beam Device to Remotely Deactivate Electronic Ordnance Devices or Terrorist Bombs.

### Overview:

The e-Zapper is a portable 20 MeV electron beam gun which produces a pencil beam with a useful range of 270 feet in air. It can be used to deactivate any electronic timing device (as might be associated with a timed bomb). The same device can be used to neutralize bio-weapons such as anthrax powder. The cost of the prototype machine should be quite low: $45k for all the device components, since it is constructed from reconditioned hospital radiation equipment (purchased new: $480k). The time for deactivation of an electronic timer is essentially instantaneous. The length of the beam gun is only 3 feet, weighing about 200 lb, making it easily transportable. The primary limitation of this proposal is possible radiation exposure, which can be overcome by making the unit remotely controlled.

### Background

A method is proposed for a non-invasive method to deactivate the electronics of unknown containers that might be considered a terrorist bomb. Non-invasive techniques would be especially useful to defeat booby-trapped parcels. Previously, the only non-invasive method pursued was a micro-wave gun which met with limited success, in part because it is relatively simple to shield against. We have investigated using other types of particle beams, and have conducted experiments with protons, neutrons, electrons and high energy photons to deactivate a bomb’s control circuits. Neither photons nor neutrons showed significant promise for reasonable exposures. Both protons and electron beams shows instant immobilization of electronics. Proton guns are impractical as a portable device; however a suitable electron gun can be packaged in a light mobile unit.

We have evaluated the use of e-beams for various electronic timing devices and have found that it is 100% effective. It has no effect on any materials other than electronics, and neutralizes timing electronics within 1 second. The device has not been tested for its effect on live explosives.

### Device Construction

The proposed device is a remotely controlled cart, similar to those used by metropolitan police forces, which carries a specialized 20 MeV electron beam generator. The concept is shown in the Figure. The cart is remotely guided to within sight of the suspected box, aligned and then activated. The cart generates an intense high-energy electron beam capable of penetrating more than a foot wood or 1½ inches of solid aluminum. Since the location of a timing device may be unknown, the gun will sweep the parcel with its beam and in a period of a minute or two the device deactivates any electronics. The weight of the cart is about 200 lbs, and associated control and power modules are about the same (see box connected with umbilical).

### Experimental Background:

During 2002, tests were made to determine the effect of 3 MeV electrons on the operation of electronic timing circuits and whether there was a threshold of electron dose that would cause the circuits to cease operation. Measurements were made at Radiation Dynamics Inc. (NY), and used three different battery-operated timers. The first test was a failure as the electron radiation destroyed the LED timer screen within the first second. Two electronic clocks with mechanically driven timing displays were then tested, and following a surface exposure of approximately 2 M-Rads at the sample, each timer stopped and could not be restarted. This is the equivalent of less than a 1 second exposure with a 10μA/cm2 electron beam. The proposed e-Zapper device will be able to deliver up to 1000x this current, and will slowly scan over the surface of the package.

Note: Using the nickname “e-Zapper” had been criticized as being flippant. But on the other hand, almost everyone who has heard about this proposal later has instant recognition when they hear the term. So for the present, we have kept the name as a useful memory tickler.

## TECHNICAL SECTION

### (1) Scientific Technical Merit

- The prototype 20 MeV e-beam machine will be built out of standard commercial components, and its operation should pose few engineering problems. Several hundred similar tools are made every year for other industrial applications, as noted in the Background. Various companies recondition hospital electron beam tools for resale, primarily to overseas customers. Such tools come with complete electronics and typically a 90 day warrantee. We propose to shrink the electronic control sections using standard COTS components. Our team has lengthy experience building accelerators using RF electronics. The USNA has particular expertise in constructing remote control vehicles as more than 30 are made each year in the Systems Engineering Dept. Remote operation of military hardware is a recognized and advanced specialty in military engineering studies.

- Special attention will be made concerning operator safety from radiation exposure. Thousands of MeV electron accelerators are in operation in factories every day, so this problem should be tractable, especially for operation in a remotely controlled vehicle. Various industries which use tools similar to the e-Zapper are: hospital radiation oncology centers, the sterilization of medical equipment, the sterilization of postal packages against bio-weapons (e.g. Brentwood postal center for Congress), and the sterilization of meat products for extended room-temperature storage.

If the beam can penetrate through to target electronics, we have shown that electronic timers can be destroyed in less than a second with a relatively low beam current, < 100 μA. It is expected that other common military electronic parts will respond similarly, except for CPLD parts, for which we have no data.

### (2) Understanding of Navy / EOD Requirements

Proposal is directed at remote and reliable deactivation of ordinance electronics without detonating associated explosives. Demonstrations will use various ESA fuses, improvised RF weapons controllers (cell phones, model airplane controllers, garage door openers, TV remotes, etc.) and various timing devices. The selection of targets to be evaluated will be made with consultation with EOD personnel. The effects of shielding materials surrounding the ordnance will be evaluated.

### (3) Potential for Field Transition

Since all components are available in mature technologies, the e-Zapper prototype creation and evaluation are not expected to provide serious engineering problems. Specification and preliminary designs for a lightweight portable device will be a major goal, with operation by two men. Radiation safety for a small unit will be an important consideration, and this will be evaluated. Most of the scientific effort will be directed in finding the correct beam conditions to efficiently deactivate all target electronics while not detonating any associated explosives.

## INTRODUCTION

This proposal is for a portable 18-23 MeV electron beam, which can be remotely moved to within 200’ of an explosive weapon and deactivate its electronic detonation device. Past experiments have indicated that the beam will deactivate timing devices at distances exceeding 200’, almost instantaneously.

This kind of beam will also deactivate bio-weapons with solid thicknesses up to 4” (water equivalent), and can be used as a neutron source for the detection of hidden explosives using various nuclear signatures (many of these techniques are outlined in “The Analysis of Explosives”, J. Yinon, Pergamon Press, 1981). These latter applications will not be an explicit part of this proposal.

##### Background

High energy electron beams may be constructed from commercial off-the-shelf components. These beams are routinely used in industrial applications that include:

- Radiation oncology, to treat deep tumors.

- Sterilization of medical equipment.

- Sterilization of postal packages (e.g. Brentwood postal center for Congress).

- Sterilization of meat products for extended room-temperature storage.

In all these cases, the electron beam is used to kill micro-organisms with exposures of a few seconds. For the USPO Brentwood center, the beam travels in air and penetrates packages to neutralize bio-weapon materials. This facility achieves this goal without significant radiation exposure to employees.

This proposal concerns the construction, demonstration and evaluation of a portable e-beam for various military applications using reconditioned hospital equipment which is available from several sources.

We have conducted experiments to demonstrate the ability of an e-beam to disrupt and destroy electronics, especially electronic timers. Using the facilities of Radiation Dynamics Co. (NY) we exposed electronic timers to 3 MeV electron beams with currents of 10 μA. The clock electronics were destroyed within a second (about 3 M-Rad of deposited energy). Since CMOS devices will cease to function with as few as 4 holes injected in their device gate oxides, it is assumed that the e-beam dislodged a few atoms and the electronic coherence of the chips was destroyed. It only takes 25 eV to displace a gate atom, so with a 3 MeV electron beam such displacements would be widespread within the device.

An e-beam of 23 MeV will travel 80 meters through air, 260 feet,. (U. S. Bureau Standards, Rpt. NBSIR 82-2550-a). The beam will travel 76 m (248’) and still have a residual energy of 3 MeV, which is what we have shown is an adequate energy to disable electronics. This final penetration can be extended to the range through other materials, so there will be a residual 3 MeV after penetration of about 30 cm (1’) of wood or 4 cm (1.5”) of aluminum.

Typical electron source, accelerator and focusing elements for 18MeV beam.

The electron accelerator shown in the figure is used in hospitals for radiation therapy. Patients are typically within one meter of this tube and are exposed to less than 100 mRem/hr for treatments of about five minutes. For the proposed deactivation of electronic devices, the beam used will be about 1% of the hospital flux, for less than 5 seconds. This should contain any radiation exposure to typical radiation-worker levels. The e-beam traveling through air will generate some x-rays, primarily from inner shell excitation of Ar atoms. However, the expected remote operation of the e-Zapper should minimize any operator exposure from the air excitation. There is no published data which could be found about the radiation created by ~20 MeV electron beams in air. Therefore the safety aspect of using this machine in air is a major portion of the proposal.

## PROJECT TIMELINE

### Phase I - Experimental Evaluation and Prototype Development (12 months)

- Our tests have shown that electronic timing devices are permanently disabled by short bursts (<1s) of MeV e- beams. Further tests will be conducted with a much wider variety of electronic parts to determine the range of effectiveness of the e-Zapper approach.

  - Assemble electronic parts from DoD sources for testing: various ESAF components, electronic timers, etc. In particular, some parts will be supplied by Dr. Thomas F. Swean, ONR, and Scott Steward, Navy EOD, Indian Head, MD. Parts will be returned to originator after completion of tests.

  - Establish safety protocols for testing parts at commercial e-beam facilities. Review protocol with Safety Office of both USNA and proposed site of experiments to assure safe experiments.

  - Establish testing protocols for experiments concerning the deactivation of various electronic parts.

  - Determine dose and energy requirements for deactivation for each electronic part. Tests may be conducted at Radiation Dynamics Co. (NY), or at M.I.T. (MA), or elsewhere as convenient.

  - Establish deactivation of various common types of electronics with integrated circuits (CMOS logic, SRAMs, DRAMs and CPLDs, etc.)

- Evaluate vendors of commercial e-beam accelerators which can be used for an e-Zapper. Determine capabilities, size and power requirements, portability and reliability.

- Evaluate suppliers of used e-beam accelerators which can be used for applications in electronic deactivation. Determine capabilities, size and power requirements, portability and reliability.

- Evaluate modifications of the accelerator electronics for operation as e-Zapper as described in this proposal. Of special concern is the effective operation of the electron accelerator at greatly reduced currents. Our tests have shown that deactivation will occur at less than 1/100th the typical available electron currents. It may be desirable to operate at these lower currents to minimize the chance of detonating nearby explosives.

- Evaluate experimental sites for the testing to be done in Phase II. Consult with Safety Officers of all sites selected to outline required precautions which must be met before beginning experiments. These discussions will be codified in Phase II as the required experimental protocols before any experiments are conducted.

- Deliver a technical report summarizing evaluations and containing technical specifications and cost information for prototype construction

During Phase I, no radiation experiments will be conducted at either USNA or NRL.

### Phase II - Prototype Construction and Testing (18 months)

- Purchase electron accelerator and electronics based on evaluations of Phase I.

- Purchase suitable portable electric generator to meet requirements of e-Zapper. Depending on the results of Phase I (part 1), this source may be a simple set of batteries (for a single e-Zapper shot) or a more elaborate motor-generator set.

- Design and build portable shock-resistant platform to mount e-Zapper equipment. An example would be to incorporate all equipment into a small trailer equipped with air-cushioned shock absorbers. This will allow portability between sites for various tests.

- Integrate accelerator, electronics, power generator and shielding into portable trailer.

- Develop radiation shielding of portable trailer to lower radiation leakage to acceptable standards.

- Develop experimental testing protocols for the portable e-Zapper operation. All protocols, and the e-Zapper shielding, must be approved by the site Safety Officer at the selected test sites (probably NRL, Indian Head, or USNA) before any of the below tests are performed. Test sites will be selected as part of Phase I objectives.

- Measure radiation leakage of e-Zapper unit under typical field conditions. Modify shielding as needed to assure safe operation.

- Measure electron beam spread (lateral straggling) as a function of distance through the air to evaluate the dilution of the central beam intensity.

- Test deactivation of electronics with prototype e-Zapper system. This step would repeat some of the experiments of Phase I, part 1d (using 3 MeV electron beams) to be sure that portable e-Zapper beam, at ~20 MeV, is equally effective.

- One major advantage of the e-Zapper is to deactivate electronics without detonating any nearby explosives. Because deactivation of electronics has been shown to require less beam energy than 1 Watt spread over a cm of depth, tests will be designed at the Navy EOD Center, Indian Head, MD, in collaboration with Dr. Philip Miller, Director of Dynamics and Diagnostics. These tests will combine deactivation of electronics, attached to various explosives, to test the localized neutralization of electronics without detonation of explosives. The selection of the types of explosives to be tested will be done in consultation with ONR and the Navy EOD Division.

- Develop interim technical report summarizing Phase II results

### Phase III - Prototype Evaluation and Field Trials (18 months)

- Evaluate radiation leakage for e-beam unit when in field operation. Correct any radiation leaks to produce a safe instrument.

- Design magnetic scanning mechanism to sweep the beam and to allow irradiation of large targets (1m x 1m).

- Conduct tests to evaluate the deactivation of electronics, attached to various explosives, to test the localized neutralization of electronics without detonation of explosives. The selection of the types of explosives to be tested will be done in consultation with ONR and the Navy EOD Division. Testing with live explosives is not included in the proposal budget.

- Evaluate feasibility of a 2-man portable e-Zapper unit (<200 lb).

- Evaluate effect of common shielding materials on deactivating electronics. Shielding will absorb the e-beam, and at some point the beam will be totally absorbed and no longer effective. The depth of such critical shielding is expected to range from a foot of wood, to 1.5 inches of aluminum (estimates from ICRU #37, by Berger and Seltzer of NIST, 1984).

- Perform additional system field evaluation of electronic deactivation on devices supplied by military

- Design system for remote control of e-Zapper, both guiding it to a target and operating the e-Zapper controls. The remote controller design will depend upon the results obtained in Phase II, especially the determination of beam current, exposure time, probability of detonating nearby explosives, and exposure of a nearby operator to radiation.

- Building a prototype of the remotely controlled e-Zapper will depend on the complexity of the requirements for this unit. This can not be determined until Phase III, part 6, is completed. Further, implementation of an advanced remotely controlled vehicle which includes ability to traverse rough ground or climb stairs may require purchase of a commercial remote-controlled carrier (not included in proposal budget).

- Develop detailed final technical report on e-Zapper project

## MANAGEMENT SECTION

### (4) USNA/NRL Capability, Experience, Facilities and Techniques

This proposal is a joint effort between the United States Naval Academy and Naval Reseach Laboratory. Both the USNA and NRL has built and operated beam accelerators for more than fifty years. They are experts in modeling radiation generation and exposure. They have world experts in the interaction of radiation with electronics. Both groups have access to special high-radiation testing areas, which can be used to safely evaluate and test e-Zapper components. Finally, they have extensive expertise in remote robotic operation of vehicles and equipment.

### (5) Qualifications of Key Team Personnel

- James Ziegler – Professor of Electrical Engineering, USNA. World expert on effects of radiation on electronics. Also a world expert on interactions of beams with matter, and transport of beams in air. More than 200 publications in this area as well as 21 books.

- Martin Nelson – Professor of Nuclear and Mechanical Engineering, USNA. Expert in radiation safety, nuclear modeling and neutron physics.

- James D. Zuber – Assistant Professor of Electrical Engineering, USNA. Expert in RF power  circuits similar to those used to drive the e-Zapper.

- Harold Hughes – Manager of Solid State Devices, Naval Research Laboratories. Forty years of research in effects of radiation on electronic devices and integrated circuits.

- Kenneth Purser – World expert on construction of beam accelerators. Founder and CEO of General Ionex Co., which sold more than $200M of RF-based accelerators from 1980-2000. Expert in RF electronics as applied to beam acceleration. Currently employed by Southern Cross Co. of Massachusetts.

- Philip Miller, Director of the Division of Dynamics and Diagnostics, Naval Explosive Ordnance Disposal Technology Division, Indian Head, MD.

## COST SECTION

### (6) Summary of Proposed Costs and Fees

Budget –Phase I  (October 1, 2004 thru September 30, 2005)

| Supplies | $6,000 |
| --- | --- |
| Equipment and Services (Beam Time) | $12,000 |
| Travel | $3,250 |
| Purchase Services-NRL | $10,000 |
| Purchase Services-Southern Cross Co. | $40,000 |
| James Ziegler ($325/day x 150 days) | $48,750 |
| Martin Nelson ($800/day x 25 days) 3 | $20,000 |
| Total phase I cost | $140,000 |

Budget –Phase II (October 1, 2005 thru March  31, 2007)

| Supplies | $12,000 |
| --- | --- |
| Equipment | $70,000 |
| Travel | $3,500 |
| Purchase Services-NRL 1 | $35,000 |
| Purchase Services-Southern Cross Co.2 | $40,000 |
| James Ziegler ($350/day x 250 days) 3 | $87,500 |
| Martin Nelson ($850/day x 20 days) 3 | $17,000 |
| James D. Zuber ($600/day x 28 ⅓ days) 3 | $17,000 |
| Technician Support | $90,000 |
| Consultation with Navy EOD Unit at Indian Head for Phase II, part 10. Does not include direct costs of tests with live explosives. | $15,000 |
| Total phase II cost | $387,000 |

Budget –Phase III  (April 1, 2007 thru September 30, 2008)

| Supplies | $9,500 |
| --- | --- |
| Equipment | $10,000 |
| Travel | $3,250 |
| Purchase Services-NRL 1 | $90,000 |
| Purchase Services-Southern Cross Co. 2 | $40,000 |
| James Ziegler ($375/day x 250 days) 3 | $93,750 |
| Martin Nelson ($900/day x 15 days) 3 | $13,500 |
| James D. Zuber ($675/day x 20 days) 3 | $13,500 |
| Technician Support | $90,000 |
| Total Phase III cost | $363,500 |

Total project cost Phases I-III   $890,500

Funds transferred to USNA by use of MIPR - Address/phone:

Comptroller’s Office-US Naval Academy, Attn: Ruth Gibbons

181 Wainwright Road

Annapolis, Maryland 21402-5008

410-293-1617

## Attachment A – CV of James Ziegler

James F. ZIEGLER

Professional Biography

Education

B.S., M.S., Ph.D.(1967) in Physics from Yale University.

Work Experience

James Ziegler joined IBM-Research in 1967.  His initial work involved ion channeling studies of impurities in crystals, ion implantation to form electronic devices and radiation damage in materials. He was involved in the introduction of integrated circuit (IC) fabrication at IBM, especially the use of ion implantation to make doped semiconductor junctions. This led to work in IC device design and the conversion of discrete component technology to equivalent IC circuit replication. His 1979 paper suggesting that cosmic rays were a substantial contributor to electronic reliability has become the benchmark in the analysis of system reliability of terrestrial electronics. From 1996-2000 he conducted research in new concepts of high-frequency communication technologies including the incorporation of MEMS concepts into integrated circuit RF designs.

He has been elected a Fellow of the International Electrical and Electronic Engineering Society (IEEE) and also a Fellow of the American Physical Society. In 1987 he was awarded the "von-Humboldt Senior Scientist Prize" by the German Government. He is the author or editor of 18 scientific books on the above topics, and holds 16 U. S. Patents, mostly in integrated circuit technology. He is an instructor in various IBM schools, including the IBM Technical Education Institute (engineering lectures on VLSI processing) and the IBM Technical Vitality Educational Institute (public scientific lectures on topics such as "Global Warming" and "Cold Fusion"). He is the Chairman of two international technical schools: The CEI Schools on VLSI Processing (Europe and Singapore), and The International School of Ion Implantation (given next in 2002).

He has been the manager of various IBM groups in the development and analysis of integrated circuit components and material science, and has published papers concerning VLSI design modeling, ion implantation, ion sputtering, ion beam lithography, micro-ion beams, ion accelerator development, the theory of ion penetration of matter; and the effects of radiation on VLSI and computer systems.

In 2000, he left IBM to teach at the United States Naval Academy, Annapolis, MD, where he teaches science to Electrical Engineers.

EXTERNAL  HONORS

·	Fellow, The American Physical Society (1980).

·	Fellow, The IEEE Society (1996).

·	Member: The Health Physics Society, The Institute of Physics

·	Awarded the "von-Humboldt Senior Scientist Prize" by the German Government (1987, $50k) for research in “The Theory of Ion Implantation”.

·	Awarded the "Ion Implantation Scientific Achievement Prize" by the International Conference on Ion Implantation (1990).

·	Awarded the "Ion Implantation Distringuished Service Award" by the International Conference on Ion Implantation Technology (2000).

·	Winner of I.R.-100 International Award for Industrial Research for "Dendritic Solar-Energy Conversion Surface" (1975).

·	Chairman, NSF Physics Panel on Small Business Innovative Research (1987-1994).

·	Board of Editors - Applied Physics Letters  (1977-1981).

·	Board of Editors - Journal of Applied Physics (1977-1981).

·	Board of Editors - Nuclear Instruments and Methods in Research in Physics (1981-   ).

·	Moshinski Professor of Physics, National University of Mexico (1975, 1978, 1980, 1982).

Author / Editor of 19 Scientific Books :

·	"Inelastic Electron Scattering by Nuclei"

288 pages,	U.S. Atomic Energy Commission (1967)

·	"Ion Beam Surface Layer Analysis"

422 pages,	Elsevier Press of North Holland (1973)

·	"New Uses of Ion Accelerators"

482 pages,	Plenum Press (1975)

·	"Hydrogen Stopping Powers and Ranges in all Elements"

317 pages, 	Pergamon Press (1977)

·	"Helium Stopping and Ranges in all Elements"

367 pages, 	Pergamon Press (1978)

·	"Bibliography of Experimental Range and Stopping Power Data"

207 pages,	Pergamon Press (1977)

·	"Handbook of Heavy Ion Stopping Cross-Sections in all Elements"

489 pages,	Pergamon Press (1980)

·	"Handbook of the Range Distributions of Energetic Ions in All Elements"

512 pages,	Pergamon Press (1980)

·	"Ion Implantation - Science and Technology"

620 pages,	Academic Press (1984)

·	"Ion Implantation Equipment and Techniques"

457 pages,	North Holland (1985)

·	"The Stopping and Range of Ions in Solids"

321 pages,	Pergamon Press (1985)

·	"Ion Implantation - Science and Technology, 2nd Edition"

501 pages,	Academic Press (1988)

·	"Ion Beam Analysis"

743 pages,	North Holland (1990).

·	"Handbook of Ion Implantation Technology"

700 pages,	IIT Press (1994).

·	"Ion Implantation - Science and Technology, 6th Edition"

727 pages,	Ion Implant Press (1998)

·	"Ion Implantation - Science and Technology, 2000 Edition"

776 pages,	IIT Press (2000)

·	"Ion Implantation - Science and Technology, 2002 Edition"

776 pages,	IIT Press (2000)

·	"The Stopping and Range of Ions in Solids"

588 pages,	IIT Press (2002)

·	"Soft Fails – History,Trends and Challenges"

422 pages,	Cypress Semiconductor Press (2004)

United States Patents :

- 3,470,305 – “Internote Coupling Means for an Electropiano”

- 3,963,172 - "Contact Metallization for Semiconductor Devices"

- 4,005,698 - "Photon Energy Converter"

- 4,053,925 - "A Method and Structure for Controlling  Carrier Lifetime in Semi-Conductor Devices"

- 4,111,720 - "A Method for Forming a Non-Epitaxial Bipolar Integrated Circuit"

- 4,110,625 - "A Method and Apparatus for Monitoring the Dose of Ions Implanted  into a Target by Counting Emitted X-rays"

- 4,528,212 - "Coated Ceramic Substrates for Mounting Integrated Circuits"

- 5,912,210 - “Enhancement of Persistent Currents in High TC Superconductors”

- 6,179,767 - “Focussing of Therapeutic Radiation on Internal Structures of Living Bodies”

- 6,230,038 - “Imaging of Internal Structures of Living Bodies by Sensing Implanted Magnetic Devices”

- 6,238,946 - Process for Fabricating Single Crystal Resonant Devices that are Compatible with Integrated Circuit Processing”

- 6,245,005 - “Method and Apparatus for Facilitating Radiation Treatment of Deep Tumors”

- 6,262,464 - “Encapsulated MEMS Band-Pass Filter for Integrated Circuits”

- 6,275,122 - “Encapsulated MEMS Band-Pass Filter for Integrated Circuits”

- 6,337,627 - “System of Providing Medical Treatment”

- 6,348,846 – “Filter circuit including system for tuning resonant structures to change resonant frequencies thereof”

(5 more patents filed and pending)

IBM - Patent Awards :

·	IBM - Invention Achievement Awards : Plateaus 1, 2, 3, 4, 5 and 6.

IBM - HONORS

IBM  - Outstanding Achievement Awards :

·	"A New Method to Profile Boron in Silicon" (1972).

·	"Investigation of Cosmic Ray Effects on Computer Memories" (1980).

·	"Development of a Krypton Screening Tool" by IBM-GTD Division (1982).

·	"Hera Project", by IBM-GTD Division (1988).

·	"Hera Project", by IBM-Research Division (1988).

IBM - Corporate Prize Awards :

·	IBM Corporate Recognition Event of 1981.

·	IBM Corporate Recognition Event of 1990.

International Conference Organizing Committees :

·	International Conference on Ion Beam Analysis (New York-1973; Germany-1975; Washington D.C.-1977; Denmark-1979; Australia-1981; Arizona-1983; Berlin-1985, South Africa-1987, Canada-1989, Netherlands-1991), Hungary-1993, Arizona-1995, Portugal-1997, Germany-1999, Australia-2001, New Mexico-2003). [Chairman in 1973 and 1990]

·	International Conference on Ion-Implantation Techniques (Salford, England-1976; Trento, Italy-1978; Kingston, Canada-1980; Berchtesgaden, Germany-1982; Vermont, USA-1984, Berkeley, CA-1986, Kyoto, Japan-1988, Surrey, England-1990, Florida, USA-1992, Catania, Italy-1994, Texas, 1996, Japan-1998, Austria-2000, New Mexico-2002). [Chairman in 1976 and 1980]

·	International Conference on New Uses of Accelerators in Science and Industry (Denton Texas- 1972; 1974; 1976; 1978; 1980; 1982; 1984; 1986; 1988, 1990, 1992).

·	International Conference on Low Energy Ion Beams (Salford, England-1976; Surrey, England-1978; Bath, England-1980).

- Pan-American Conference on Ion-Solid Interactions ( Mexico City, Mexico-1980, Chairman).

- International Conference on Ion Sources and Ion Assisted Techniques (Kyoto, Japan-1983; Strasbourg, France-1985).

·	International Conference on Microanalysis using Charge Particles (Namur, Belgium-1981).

·	Chairman, International School on Ion Implantation, (Trento, Italy-1978; Kingston, Canada-1980; Berchtesgaden, Germany-1982; Vermont, USA-1984, Berkeley, CA-1986, Kyoto, Japan-1988, Shanghai, China, 1988, England-1990, Gainesville, FL-1992).

·	Chairman, National Science Foundation Physics Committee on "Small Business Innovation Research" (1987-   ).

Scientific Reviewer for the following publications :

·	Physical Review / Physical Review Letters

·	Journal of Applied Physics / Applied Physics Letters

·	IEEE Transactions on Nuclear Science

·	IEEE Transactions on Electron Devices

·	Nuclear Instruments and Methods

·	Surface Science

·	Health Physics

·	Radiation Effects

·	Thin Solid Films

·	The Journal of Analytic Chemistry

Scientific Publications :

J. Ziegler is the author of 23 papers published in Applied Physics Letters or Physical Review Letters.

J. F. Ziegler and G. A. Peterson, "Excitation of Pb206 , Pb207 , Pb208 , and Bi209 by Inelastic Electron Scattering," Phys. Rev., 165, 1337 (1968).

J. F. Ziegler, "The Calculation of Inelastic Electron Scattering by Nuclei," 280 pages, USAEC Tech. Info. Book Yale 2726E-49.

J. F. Ziegler, "Experimental Evaluation of High Energy Ion Implantation Gradients for Possible Fabrication of a Transistor Collector," IBM J. of Research and Development, 15, 452 (1971).

M. H. Brodsky, D. M. Kaplan and J. F. Ziegler, "The Optical Absorption of Amorphous Silicon as a Function of Decreasing Spin Concentration," Proc. 11th Conf. on the Physics of Semiconductors (Warsaw, 1972).

James F. Ziegler, "Determination of Lattice Disorder Profiles in Crystals by Nuclear Backscattering," J. Applied Physics 43, 2973 (1972).

James F. Ziegler, "Determination of Surface Impurity Concentration Profiles by Nuclear Backscattering," Applied Physics Letters 20, 178 (1972).

J. F. Ziegler and B. L. Crowder, "Defining the 'Random' Spectrum as Used in the Channeling Technique of Backscattering," Applied Physics Letters 20, 178, (1972).

M. H. Brodsky, D. Kaplan and J. F. Ziegler, "Densities of Amorphous Si Films by Nuclear Backscattering," Applied Physics Letters 21, 305 (1972).

J. F. Ziegler, M. Berkenblit, T. B. Light, K. C. Park and A. Reisman, "Nuclear Backscattering Analysis of Nb / Nb2O5 / Bi Structure," IBM J. of Research and Development 16, 530 (1972).

J. F. Ziegler, G. W. Cole and J. E. E. Baglin, "Technique for Determining Concentration Profiles of Boron Impurities in Substrates," J. Applied Physics 43, 3809 (1972).

J. F. Ziegler, G. W. Cole and J. E. Baglin, "Discovery of Anomalous Base Regions in Transistors," Applied Physics Letters 21, 177 (1972).

J. F. Ziegler, B. L. Crowder, G. W. Cole, J. E. Baglin and B. J. Masters, "Boron Atom Distributions Obtained by Ion Implantation Doping of Silicon," Applied Physics Letters 21, 16 (1972).

C. J. Kircher, J. W. Mayer, K. N. Tu and J. F. Ziegler, "Analysis of the Formation of Hafnium Silicides on Silicon," Applied Physics Letters 22, 81 (1973).

J. F. Ziegler and M. H. Brodsky, "The Specific Energy Loss of He4 Ions in Silicon (Amorphous, Polycrystalline, and Single Crystal)," J. Applied Physics 44, 188 (1973).

W. K. Chu and J. F. Ziegler, "Energy Loss Measurements of He4 Ions in Heavy Metals," Applied Physics Letters 22, 437 (1973).

W. K. Chu, B. L. Crowder, J. W. Mayer and J. F. Ziegler, "Range Distribution of Implanted Ions in SiO2 , Si3 and Al2O3," Applied Physics Letters 22, 490 (1973).

J. W. Mayer, J. F. Ziegler, L. L. Chang, R. Tsu and L. Esaki, "Profiling of the Periodic Structure (GaAs/GaAlAs) by Nuclear Backscattering," J. Applied Physics 44, 2322 (1973).

C. J. Kircher, J. W. Mayer, K. N. Tu and J. F. Ziegler, "Kinetics of the Formation of Hafnium Silicides on Silicon," J. Applied Physics 44, 3851 (1973).

K. N. Tu, J. F. Ziegler and C. J. Kircher, "Formation of Vanadium Silicides by the Interactions of V with Bare and Oxidized Silicon," Applied Physics Letters 23, 493 (1973).

B. L. Crowder, J. F. Ziegler and F. F. Morehead, "The Application of Ion Implantation to the Study of Diffusion of Boron in Silicon," Ion Implantation, B. L. Crowder, ed., Plenum Press (1973) pg. 267.

Billy L. Crowder and James F. Ziegler, "The Influence of the Amorphous Phase on Boron Atom Distributions in Ion Implanted Silicon," ibid. pg. 257.

W. K. Chu, B. L. Crowder, J. W. Mayer and J. F. Ziegler, "Ranges and Distribution of Ions Implanted in Dielectrics," ibid, pg. 243.

J. Ziegler and W. K. Chu, "Stopping Cross Sections and Backscattering Factors for He4 Ions in Matter," Atomic Data and Nuclear Data Tables 13, 463 (1974).

R. J. Gambino, J. Ziegler and J. J. Cuomo, "Effects of Ion Radiation Damage on the Magnetic Domain Structure of Amorphous Gd-Co Alloys," Applied Physics Letters 24, 99 (1974).

J. E. E. Baglin and J. F. Ziegler, "Test of Bragg Rule for Energy Loss of He4 Ions in Solid Compounds," J. Applied Physics 45, 1413 (1974).

J. F. Ziegler, J. E. E. Baglin and A. Gangulee, "Thermal Stability of a Proposed Magnetic Bubble Metallurgy," Applied Physics Letters 24, 36 (1974).

J. F. Ziegler and J. E. E. Baglin, "Does Backscattering of He4 Ions by Nuclei Obey the Rutherford Cross-Section Formula?" J. Applied Physics 45, 1888 (1974).

J. E. E. Baglin, V. Brusic, E. Alessandrini and J. Ziegler, "Thin Film Interdiffusion of Chromium and Copper," Applications of Ion Beams to Metals, 169, Plenum Press (1974).

R. Hasegawa, R. J. Gambino, J. J. Cuomo and J. F. Ziegler, "Effect of Thermal Annealing and Ion-Radiation on the Coercivity of Amorphous Gd-Co Films," Applied Phys. Letters 24, 99 (1974).

J. F. Ziegler, "Calculations of Recoils of Elastically Scattered He4 Projectiles," Thin Solid Films 19, 289 (1973).

J. F. Ziegler and R. F. Lever, "Calculations of Elastic Scattering of He4 Projectiles from Thin Layers," Thin Solid Films 19, 291 (1973).

J. F. Ziegler and W. K. Chu, "The Stopping of He4 Ions in Elemental Matter," Thin Solid Films, 19, 281 (1973).

J. Meyer, J. F. Ziegler, ed., "Ion Bean Surface Layers Analysis," 463 pages, Elsevier Sequoia Press, Lausanne, 1974.

A. L. Lurio, J. A. Cairns and J. F. Ziegler, "Characteristic and Non-Characteristic X-Rays Produced During 1 MeV Argon Bombardment of Silicon," Phy. Rev. A12, 498 (1975).

Y. J. van der Meulen, C. M. Osburn and J. F. Ziegler, "Properties of SiO2 Grown in the Presence of HCL or Cl2, J. of the Electrochemical Society 122, 284 (1975).

J. J. Cuomo, J. F. Ziegler and J. Woodall, "A New Concept for Solar Energy Thermal Conversion," Applied Physics Letters 26, 557 (1975).

W. K. Chu and J. F. Ziegler, "An Analytic Solution to Elastic Nuclear Backscattering," J. of Applied Physics 46, 2768 (1975).

W. K. Chu, V. L. Moruzzi and J. F. Ziegler, "Calculations of the Energy Loss of He4 Ions in Solid Elements," J. of Applied Physics 46, 2817 (1975).

J. F. Ziegler, W. K. Chu and J. S. -Y. Feng, "Evidence of Solid State Effects in the Energy Loss of He4 Ions in Matter," Applied Physics Letters 27, 387 (1975).

R. B. Laibowitz, C. C. Tseui, J. J. Cuomo, J. F. Ziegler and M. Hatzakis, "Fabrication and Josephson Properties of Nb3Ge" IEEE Trans. on Magnetism MAG-11, 883 (1975).

J. M. Blum, J. C. McGroddy, P. G. McMullin, K. K. Shih, A. W. Smith, and J. F. Ziegler, "Oxygen Implanted Double Heterojunction GaAs/GaAlAs Injection Lasers, IEEE J. of Quantum Electronics QE-11, 413 (1975).

W. Reuter, A. Lurio, F. Cardone and J. F. Ziegler, "The Quantitative Analysis of Complex Targets by Proton Induced X-Rays," J. of Applied Physics 46, 3194 (1975).

J. F. Ziegler, ed., "New Uses of Ion Accelerators," 525 pages, Plenum Press (1975).

J. F. Ziegler, "Material Analysis Using Ion Beams," Physics Today, November, 1976.

J. F. Ziegler and W. K. Chu,  "Tables of Stopping Cross-Sections for He4 Ions in Solids," J. Appl. Physics 47, 2239 (2976).

J. F. Ziegler, W. K. Chu and J. S. -Y. Feng, "Evidence of Solid State Effects in the Energy Loss of He4 Ions in Matter," pg. 25, Ion Beam Surface Layer Analysis, Vol. I, Plenum Press (1976).

J. F. Ziegler, R. F. Lever and J. K. Hirovonen, "Computer Analysis of Nuclear Backscattering," ibid, pg. 163.

J. F. Ziegler, "Review of Trace Analysis of Ion Induced X-rays," ibid, pg. 759.

J. A. Cairns, A. Lurio, J. F. Ziegler, D. F. Holloway and J. Cookson, "The Use of Proton Induced X-Rays to Monitor the Near Surface Composition of Catalysts," ibid, pg. 773.

J. F. Ziegler, "Recent Developments in Ion Beam Analysis,"

Jour. Vac. Sci. and Tech. 13, 410 (1976).

A. Lurio and J. F. Ziegler, "Charge States of Backscattered He Ions," "Beam Foil Spectroscopy," Vol. 2., pg. 665, Plenum Press (1976).

J. F. Ziegler and W. K. Chu, "Tables of Stopping Cross-Sections for He4 Ions in J. Appl. Phys. 47, 2239 (1976).

W. A. Lanford, H. P. Trautvetter. J. F. Ziegler and J. Keller, "New Precision Technique for Measuring the Concentration Versus Depth of Hydrogen in Solids," Appl. Phys. Lett. 28, 566 (1976).

J. F. Ziegler and I. V. Mitchell, "Ion Induced X-ray Analysis," "Ion Beam Analysis," pages (X-1)-(X-137), National Science Foundation, Washington, (1976).

R. K. Nesbet and J. F.  Ziegler, "Low-velocity Stopping of Heavy Ions," Applied Physics Letters 31, 810 (1977).

J. F. Ziegler, "The Electronic and Nuclear Stopping of Energetic Ions," Applied Physics Letters 31, 550 (1977).

A. Lurio and J. F. Ziegler, "X-rays Produced during Ion Implantation," Applied Physics Letters 31, 482 (1977).

J. A. Cairns, C. W. Keep, R. J. Waite, J. F. Ziegler and K. N. Tu, "A Controlled Atmosphere Electron Microscopy Study of the Influence of Oxygen on the Interdiffusion of Gold and Iron," Applied Physics Letters 30, 392 (1977).

J. F. Ziegler, "Profiling H and He in Materials Using Ion Beams," Nuclear Society Transactions 26, 146, (1977).

M. H. Brodsky, M. A. Frisch, J. F. Ziegler, and W. A. Lanford, "Quantitative Analysis of Hydrogen in Glow Discharge Amorphous Silicon," Applied Physics Letters 30, 561 (1977).

J. F. Ziegler, "Material Analysis Using Ion Beams," IBM-I.T.L. Conference on Material Analysis 89 (1977).

J. A. Cairns, A. Lurio and J. F. Ziegler, "Determination of the Near-Surface Elemental Composition of Catalysts by the Technique of Proton-Induced X-ray Analysis," Journal of Catalysis, 45, 6 (1976).

J. F. Ziegler and J. J. Cuomo, "Reduction of Ion Sputtering Yield by Special Microtopography," Applied Physics Letters 30, 268 (1977).

J. F. Ziegler, ed., "Hydrogen Stopping Powers and Ranges in All Elements," Pergamon Press, New York (1977), 317 pages, Volume 3 of "The Stopping and Ranges of Ions in Matter".

J. F. Ziegler, "Bibliography and Index of Experimental Range and Stopping Power Data," Pergamon Press, New York (1977), 207 pages, Volume 2 of "The Stopping and Ranges of Ions in Matter".

J. F. Ziegler, "Helium Stopping Powers and Ranges in All Elements," Pergamon Press, New York (1978), 372 pages, Volume 4 of "The Stopping and Ranges of Ions in Matter."

A. Lurio, J. F. Ziegler and J. J. Cuomo, "A New Method for the Determination of Low-Energy Stopping Powers," Nuclear Instruments & Methods 149, 155 (1978).

J. F. Ziegler, "The Calculation of Low Energy He Ion Stopping Powers," Nuclear Instruments & Methods 149, 129 (1978).

J. F. Ziegler, C. P. Wu, P. Williams, C. W. White, B. Terreault, B. M. U. Scherzer, R. L. Schulte, E. J. Schneid, C. W. Magee, E. Ligeon, J. L. 'Ecuyer, W. A. Lanford, F. J. Kuehne, E. A. Kamykowski, W. O. Hofer, A. Guivarc'h, C. H. Filleux, V. R. Deline, C. A. Evans, Jr., B. L. Cohen, G. J. Clark, W. K. Chu, C. Brassard, R. Blewer, R. Behrisch, B. R. Appleton and D. D. Allred, "Profiling Hydrogen in Materials Using Ion Beams," Nuclear Instruments & Methods 149, 19 (1978).

J. Rickards and J. F. Ziegler, "Trace-Element Analysis of Water Samples Using 700-keV Protons," Applied Physics Letters 27, 707 (1978)

M. J. Nass, A. Lurio and J. F. Ziegler, "Computer Analysis of X-ray Spectra," Nucl. Inst. and Meth. 154, 567 (1978).

G. J. Iafrate and J. F. Ziegler, "Numerical Evaluation of Lindhard's Theory of Stopping Power for a Charged Particle in a Free-electron Gas," Jour. of Appl. Phys. 50, 5579 (1979).

J.A. Cairns, A. Lurio and J. F. Ziegler, "A Method and Apparatus for Monitoring the Dose of Ions Implanted into a Target by Counting Emitted X-rays.," U.S. Patent 4,110,625.

A. E. Michel, R. O. Schwenker and J. F. Ziegler, "A Method for Forming a Non-Epitaxial Bipolar Integrated Circuit," U. S. Patent 4,111,710.

J. F. Ziegler and W. A. Lanford, "The Effect of Cosmic Rays on Computer Memories," SCIENCE, Vol. 206, 776 (1979).

J. F. Ziegler and W. A. Lanford, "The Effect of Sea Level Cosmic Rays on Electronic Devices," IEEE 1980 International Solid-State Circuits Conference Proceedings, Vol. XXII, 70 (1980).

G. J. Iafrate and J. F. Ziegler, "Application of Lindhard's Dielectric Theory to the Stopping of Ions in Solids," J. Appl. Phys. 51, 984 (1980).

J. F. Ziegler and G. J. Iafrate, "The Stopping of Energetic Ions in Solids," Radiation Effects 46, 199 (1980).

J. F. Ziegler, "Handbook of Stopping Cross-Sections for Energetic Ions in All Elements," Vol. 5 of the Series: "The Stopping a Ranges of Ions in Matter," Pergamon Press, New York (1980), 432 pages.

U. Littmark and J. F. Ziegler, "Handbook of Range Distributions for Energetic Ions in All Elements," Vol. 6 of the Series: "The Stopping and Ranges of Ions in Matter," Pergamon Press, New York (1980), 498 pages.

U. Littmark and J. F. Ziegler, The Ranges of Energetic Ions in Matter," Physical Review 23A, 64 (1981).

W. A. Lanford and J. F. Ziegler, "Computers," Chapter in the McGraw-Hill Yearbook of Science and Technology, pp. 142 (198l).

J. F. Ziegler, "The Effect of Concrete Shielding on Cosmic Ray Induced Soft Fails in Electronic Systems," I.E.E.E. Trans. on Electron Devices ED-28, 560 (1981).

J. F. Ziegler and W. A. Lanford, "The Effect of Sea Level Cosmic Rays on Electronic Devices," Jour. Appl. Physics 52, 4305 (1981).

J. F. Ziegler, "The Background in Detectors Caused by Sea Level Cosmic Rays," Nuclear Inst. and Meth. 191, 419 (1981).

J. P. F. Sellschop and J. F. Ziegler, "Background in Semiconductor Devices," Nuclear Inst. and Meth. 191, 447 (1981).

J. P. Biersack and J. F. Ziegler, "Refined Universal Potentials in Atomic Collisions," 194, 93 (1982).

J. F. Ziegler and J. P. Biersack, "The Stopping and Ranges of Ions in Solids," Chapter in "Ion Implantation Techniques," Springer-Verlag Series in Electrophysics #10, (1982).

J. P. Biersack and J. F. Ziegler, "The Calculation of Ion Ranges in in Solids with Analytic Solutions," Chapter in "Ion Implantation Techniques," Springer-Verlag Series in Electrophysics #10, (1982).

J. F. Ziegler, C. C. Tsuei, C. C. Chi, C. D. Tesche, P. Chaudhari and K. W. Jones, "Test of a Superconducting Magnetic-Monopole Detector for Spurious Events Due to Sea Level Cosmic Rays," Phys. Rev. D18, Rap. Commun., 1793 (1983).

J. F. Ziegler, J. P. Biersack and U. Littmark, "The Theory of Stopping

Powers for Ions in Solids," Proc. of Int'l. Ion Engineering Congress, ISIAT-83' and IPAT-83', Kyoto, Japan, pp. 1861-1874 (1983).

J. F. Ziegler, J. P. Biersack and U. Littmark, "Empirical Stopping Powers for Ions in Solids," Proc. of U.S.-Japan Seminar on Charged-Particle Penetration Phenomena," U.S.-Nat'l. Science Found. Conf. No. 82013 (1983).

J. Biersack and J. F. Ziegler, "The Stopping and Range of Ions in Solids," Chapter in "Ion Implantation Techniques," Vol. 10 of Springer Series in Electrophysics (1983).

J. E. E. Baglin, J. A. Cairns, G. J. Clark and J. F. Ziegler, "Interaction of Al2O3 with Rh and Pt Thin Films," J. Molecular Catalysis, 20, 299 (1983).

P. A. Saunders and J. F. Ziegler, "Interactive Computer Analysis of Nuclear Backscattering Spectra," Nuclear Instruments and Methods, 218, 67 (1984).

J. E. E. Baglin, G. J. Clark and J. F. Ziegler, "Catalyst-Metal Support Interactions:  Rutherford Backscattering Spectrometry Applied to Discontinuous Films," Nuclear Instruments and Methods, 218, 445 (1984).

J. F. Ziegler and J. P. Biersack, "The Stopping and Range of Ions in Solids," Part II of the book "Ion Implantation Techniques," Springer-Velag, Berlin (1984).

J. F. Ziegler and R. F. Lever, "The Channeling of Ions Near the Silicon  <001>  Axis," Applied Physics Letters, 46, 358 (1985).

J. F. Ziegler, "The Stopping and Range of Ions In Solids," Chapter 2 of "Ion Implantation - Science and Technology," Academic Press, New York (1984).

J. F. Ziegler, "High Energy Ion Implantation," Nuclear Instruments and Methods, B6, 270 (1985).

J. F. Ziegler and R. L. Brown, editors of "Ion Implantation Equipment and Techniques," 457 pages, North Holland Pub. Co., Amsterdam (1985).

J. F. Ziegler, "Ion Implantation - Science and Technology," 620 pages, Academic Press (1985).

J. F. Ziegler, J. P. Biersack and U. Littmark, "The Stopping and Range of Ions in Solids," 321 pages, Pergamon Press, New York (1985).

J. F. Ziegler, editor, "Ion Implantation: Science and Technology, 2nd Edition", Academic Press (1988), 501 pages.

J. F. Ziegler and T. Zabel, "The Physics of Radioactive Materials", Chapter 1, "The IBM Hera Problem", report of the Hera Task Force of IBM - GTD (1988).

V.  Brusic, G.  Frankel, J.M.  Manoyan, T.  Zabel, J.F.  Ziegler, D. Falcon, and S.  Fridmann: "Spontaneous Plating of Polonium", IBM Confidential Report, RC-13945, February 1988.

J. F. Ziegler, "The Stopping and Range of Ions in Solids", Chapter 1 in "Ion Implantation: Science and Technology", Academic Press (1988).

J. F. Ziegler, "Pilgrim 6 - The Soft Error Rate of the IBM Bipolar Circuit Family : ATX -1", IBM Confidential - Restricted.

J. F. Ziegler and J. Manoyan, "The Stopping of Ions in Compounds", Nucl. Inst. and Meth., Jan. 1989.

J.F. Ziegler, T.H. Zabel, J.J. Cuomo, V.A. Brusic, G.S. Cargill, III, E.J. O'Sullivan and A.D. Marwick: "Electrochemical Experiments in Cold Nuclear Fusion", Phys. Rev. Lett. vol. 62, 2929 (1989)

J. F. Ziegler and T. Zabel, "The Physics of Radioactive Materials", Chapter 1, "The IBM Hera Problem", report of the Hera Task Force of IBM - GTD (1989), IBM - Confidential (Restricted).

J. F. Ziegler, "Pilgrim 7 - The Soft Error Rate of the IBM Bipolar Circuit Family : ATX - 4", IBM Confidential - Restricted.

J. F. Ziegler and J. Manoyan, "The Stopping of Ions in Compounds", Nuclear Instruments and Methods, B35, 215 (1989).

V. Brusic, G. Frankel, J.M. Manoyan, T. Zabel, J.F. Ziegler,

D. Falcon, and S. Fridmann:  "Spontaneous Plating of Polonium".  IBM Confidential Research Report  RC 13945 (#61108).

J. F. Ziegler, C. W. Curtis, H. P. Muhlfeld, C. J. Montrose, W. B. Chin, L. B. Freeman, C. A. Russell, W. Y. Wang, L. E. LaFave, J. L. Walsh, T. J. O'Gorman, A. J. Sykes, T. S. Scott and A. Taber, "Pilgrim -7, Accelerated Testing of ATX-4" (IBM - Confidential, Restricted).

J. F. Ziegler,  H. W. Curtis, H. P. Muhlfeld,  C. J. Montrose, W. B. Chin,  L. B. Freeman, C. A. Russell,  W. Y. Wang, J. M. Orro,  G. J. Unger,  J. M. Ross, L. E. Lafave,  J. L. Walsh, T. J. O'Gorman, A. J. Sykes, B. Messina T. S. Scott, A. H. Taber, R. J. Sussman, and   W. A. Klein, "PILGRIM -8 : Soft Errors from Cosmic Rays for 1989 / 1990 IBM Chips", (IBM - Confidential Report).

J. F. Ziegler, H. W. Curtis, H. P. Muhlfeld,  C. J. Montrose, W. B. Chin,  L. B. Freeman, P. Hosier, C. A. Russell,  W. Y. Wang, M. Nicewicz, J. M. Orro,  G. J. Unger,  J. M. Ross, L. E. LaFave  and  J. L. Walsh, T. J. O'Gorman, T. Sullivan, A. J. Sykes, B. Messina, V. Tolat, T. Enger, H. Yourke, T. S. Scott, A. H. Taber, R. J. Sussman and W. A. Klein, "SOFT  ERRORS  from  COSMIC  RAYS - Historical Review, Analysis of IBM Field Fails, and the Accelerated Testing of Chips" (IBM-Confidential).

J. F. Ziegler and T. Zabel, "The Physics of Radioactive Materials", Chapter 1, "The IBM Hera Problem", report of the Hera Task Force of IBM - GTD (1990), IBM - Confidential (Restricted).

C. Park, K. M. Klein, A. F. Tasch and J. F. Ziegler, "Critical Angles for Channeling of Boron Ions Implanted into Single-Crystal Silicon", J. Electrochem. Soc., vol. 138, 2107 (1991).

J. F. Ziegler, T. H. Zabel and H. Curtis, "Video Display Terminals and Radon", Health Physics Journal, vol. 65, 252-264 (1993).

J. F. Ziegler, ed., "Handbook of Ion Implantation Technology", 700 pages, Elsevier Press (1992).

W. B. Chin,  L. B. Freeman, J. M. Orro,  G. J. Unger,  J. M. Ross, L. E. LaFave, T. J. O'Gorman, T. Sullivan, A. J. Sykes, B. Messina, T. S. Scott, "Soft Errors from Cosmic Rays - T-1 Testing Results for ATX-4sx and Vendor Chips", IBM-Confidential report (1992).

- F. Ziegler, “Ion Implantation - Science and Technology, 5th Edition” , 762 pages, Ion Beam Technology Press, New York (1996).

J. F. Ziegler, "Terrestrial Cosmic Rays”, IBM Journal of Research and Development, 40 (1), 19 (1996).

J. F. Ziegler, P. A. Saunders and T. H. Zabel, "Portable Faraday Cup for Non-Vacuum Proton Beams IBM Journal of Research and Development, 40 (1), 73 (1996).

L. C. Freeman and J. F. Ziegler, "Critical Charge Calculations for a Bipolar Array Cell", IBM Journal of Research and Development, 40 (1), 119 (1996).

J. F. Ziegler, H. P. Muhlfeld, C. J. Montrose, H. W. Curtis, T. J. O'Gorman and J. M. Ross, "Accelerated Testing for Cosmic Soft-Error-Rate", IBM Journal of Research and Development, 40 (1), 51 (1996).

J. F. Ziegler, H. W. Curtis, H. P. Muhlfeld, C. J. Montrose, B. Chin, M. Nicewicz, C. A. Russell, W. Y. Wang, L. B. Freeman, P. Hosier, L. E. LaFave, J. L. Walsh, J. M. Orro, G. J. Unger, J. M. Ross, T. J. O'Gorman, B. Messina, T. Sullivan, A. J. Sykes, H. Yourke, T. Enger, V. Tolat, T. S. Scott, A. H. Taber, W. A.  Klein, and C. W. Wahaus, "IBM Experiments in Soft Fails in Computer Electronics, (1978-1994)", IBM Journal of Research and Development, 40 (1), 3 (1996).

J. F. Ziegler, M. E. Nelson, J. D. Shell, R. J. Peterson, C. J. Gelderloos, H. P. Muhlfeld and C. J. Montrose, “Cosmic Ray Soft Error Rates of 16Mb DRAM Memory Chips”, IEEE J. Solid State Circuits, 33, 246 (1998).

J. F. Ziegler, “Terrestrial Cosmic Ray Intensities”, IBM Journal of Research and Development, 42 (1), 117 (1998).

J. F. Ziegler, “Ion Implantation - Science and Technology, 6th Edition” , 762 pages, Ion Beam Technology Press, New York (1998).

G. J. Hofman, R. J. Peterson, C. J. Gelderloos, R. A. Ristinen, M. E. Nelson, A. Thompson, J. F. Ziegler and H. Muhlfeld, “Light-Hadron Induced SER and Scaling Relations for 16- and 64-Mb DRAMS“, IEEE Trans. Nucl. Sci., 47, 403-407 (2000).

M. E. Nelson, J. A. Sarlese, J. F. Ziegler, H. P. Muhlfeld, R. J. Peterson, A. Campbell, J. S. Melinger, K. Clark and A. K. Thompson, J. Rad. Effects, 19, 100-106, 2000.

R.J. von Gutfeld, J. F. Dicello, S. J. McAllister, J. F. Ziegler, “Amorphous magnetic wires for medical locator applications”. Applied Physics Letters, vol.81, no.10, 2 Sept. 2002. p. 1913-1915.

J. F. Ziegler, “Electronic Reliability from Radiation”, , NASA Workshop on Electronic Reliability of Terrestrial Systems, Proceedings, 18-27 (2003), Jet Propulsion Laboratory, Pasadena, CA.

J. D. Dirk, M. E. Nelson, J. F. Ziegler, A Thompson, T. H. Zabel, “Terrestrial Thermal Neutrons“, IEEE Trans. Nucl. Science, 50, 2060-2064 (2003).

J. F. Ziegler, “SRIM 2003”, Nucl. Instr. and Meth., 219-220, 1027-1036 (2004).

J. F. Ziegler and H. Puchner, “Soft Error Rates – History, Trends and Challenges”, 422 pages, Cypress Semiconductor Co., San Jose, CA (2004).

J. F. Ziegler, ed., “Ion Implantation – Science and Technology, 8th Edition”, 525 pages, Ion Beam Press, (2004).

## Attachment B – CV of Martin Nelson

Phone number:  410-293-6427

Address:   US Naval Academy

590 Holloway Road

Annapolis, Maryland 21402

CURRICULUM VITAE

1.	NAME:		Martin E. Nelson

2.	DEPARTMENT:	Mechanical Engineering

3.	ACADEMIC RANK:	Professor - United States Naval Academy

Adjunct Professor - University of Maryland, 						            Nuclear Engineering Department

4.	YEARS TEACHING EXPERIENCE:	(USNA/TOTAL) 33/33

5.	EDUCATION:

B.S. Applied Math and Engineering Physics, 1966, University of Wisconsin

M.S.E, Nuclear Engineering, 1968, University of Virginia

Ph.D., Nuclear Engineering, 1972, University of Virginia

6.  RECENT JOURNAL PUBLICATIONS (1998-2003)

Dirk,J., NELSON,M., Thompson, A., Zabel, T.,Ziegler, J., “Terrestrial Thermal Neutrons”, IEEE Transactions of Nuclear Science,vol 50, no. 6, pp 2060-2064, December, 2003.

McMarr, P., NELSON, M., Hughes, H., Delikat,K., “14 MeV Neutron and Co-60 Gamma Testing of a Power MOSFET Optocoupler”, IEEE Transactions of Nuclear Science, vol 50, no. 6, 2030-2037, December, 2003.

McClellan, Y., August, A., Gosz, J., Gann, S., Parmenter,R ., NELSON, M., Harper, M., “Plant and Environmental Interactions”, Journal of Environmental Quality, vol 32, pp 1759-1763,September-October, 2003.

NELSON,M.E., Sarlese, J.A., Ziegler, J.F.., Muhlfeld,h., Peterson, R.J., Campbell, A., Melinger, J.S., Clark, K., Thompson, A.K. ., “Cosmic Ray Induced Soft Error Rates of Commercial 16 Mb and 64 Mb DRAMS at Aircraft Altitudes”, Journal of Radiation Effects, volume 19-1,2001

Hofman, G.J., Peterson, R.J., Gelderloss, C.J. , Ristinen, R.A. , NELSON, M.E., Thompson, A., Ziegler, J.F., Mullfeld, H.  “Light-hadron Induced SER and Scaling Relations for 216 Mb and 64 Mb dRAMS,” IEEE Transaction on Nuclear Science, pp 1-5, 1999.

August, R. A., Philips, G.A., Harper, M., NELSON, M., Gann, S.   “Mixed Waste Characterization in Soil,” Nuclear Instruments and Methods in Physics Research, (Section A), A422, pp.767-772, February, 1999.

Millet, M, NELSON, M., Harper, M. J. “Weight and Counting Efficiency Optimization in a Moderated Neutron Detector,” Nuclear Instruments and Methods in Physics Research (Section A),  A417, pp 332-341, November, 1998.

Ziegler, J.F., NELSON, M.E., Shell, J.D., Peterson, R.J., Gelderloss, C.J., Muhfeld, H. P., Montrose, C.J.,   “Cosmic Ray Soft Error Rates of 16- Mb DRAM Memory Chips”, IEEE                             Journal of  Solid-state Circuits, vol. 33., no.2, pp. 246-252, February, 1998.

7. RECENT CONFERENCE PRESENTATIONS (2000-2004)

NELSON,M., Ziegler,J.,Fiegel,H., Good, C., Blackburn,B.,St. John,J.,  Locke,S., Allen, J., “Investigation into the feasibility of highly enriched uranium detection in a large shipping container at a US PORT using active neutron interrogation” , AIW Workshop, Idaho Falls, June, 2004

Pisacane, V.L, DiCello, J.F., Rosenfeld, A.B., Ziegler, J.F, NELSON, M.E., “MIDN: A Spacecraft Microdosimetry Mission”, 2004 Solid State Dosimetry Conference,New Haven Connecticut, June, 2004.

Lawrence,R., Jenkins,W.,Haddad, N.,NELSON, M., “14 MeV Neutron Testing of BAE Quantum and Quantum Shrink 4M SRAM Technologies”, 22nd Annual HEART Conference, Monterey, California, March, 20

Hughes, H.L., McMarr,P.J., Lawrence,R.K.,Liu,S.T., NELSON, M.E., Thompson, A.K., “Feasibility of Neutron detection Using a Boron-10 Loaded Silicon-on-Insulator Substrate for Unattended Applications”, 2004 GOMAC Conference, Monterey, california, March, 2004.

Liu,S.T.,Hughes,H. L., Jenkins, W.C., Lawrence, R.K., NELSON, M.E., Thompson, A.K, “Thermal Neutron Single Event Effects in SOI CMOS 4M SRAMS”, 2004 HEART Conference, Monterey, California, March, 2004.

Dirk,J.,NELSON,M.,Thompson,A.,Zabel,T.,Ziegler,J., “Terrestrial Thermal Neutrons”, 2003 NESREC Conference, Monterey, California, July, 2003.

McMarr,P.,NELSON, M.,Hughes,H., Delikat,K., “14 MeV Neutron and Co-60 Gamma Testing of a Power MOSFET Optocoupler”, 2003 NESREC Conference, Monterey, California, July, 2003.

NELSON, M., Dormo, J., St. John, T., Jenkins, R., “Comparison of Light Sources for Use in a Commercial TLD Reader”, 48th Annual meeting of Health Physics Society, San Diego, California, July, 2003.

NELSON, M., Messner,G., Jenkins, B., Cassata, J.,  “Development of a Temperature Stabilized Light Source for TLD”, 47th Annual Meeting of Health Physics Society, Tampa Florida, June, 2002.

Cassata, J., Ruesch,E., Pieski,E., Laird, R.,NELSON,M., Kennemur,L. , “Development of a Neutron Dose Algorithm for an Albedo Whole Body TLD that Possesses Non-Uniform Filtration over Neutron Sensitive and Non-Sensitive Element”, Health Physics Society, Cleveland, Ohio, June, 2001.

Haire, T. W., NELSON, M.E.,Cassata, J.R., Riel,G.K. , “Investigation of Photon Spectra and Light Efficiency in a LiF TLD”, Health Physics Society, Denver, Colorado, June 26, 2000.

Parrella, M.C., NELSON, M.E., Cassata, J.R., Riel, G.K., “Development of an MCNP Model for the Neutron Response of a Special Area Monitor”, Health Physics Society, Denver, Colorado, June 26, 2000.

8.     PROFESSIONAL SOCIETIES AND TITLES

1. American Nuclear Society

2. Health Physics Society

3. Registered Professional Engineering in State of Maryland

9. CURRENT RESEARCH INTERESTS

His current research interests include health physics, environmental remediation, nuclear instrumentation, nuclear arms control technologies, radiation effects on microchips, and radiation accelerators.  Dr. Nelson has served as the directors of the USNA nucleonics laboratory in Rickover Hall for over 30 years.

Specific NRL personnel will be identified based on results of previous Phase I. Primary NRL participation is expected to relate to testing deactivation of DoD supplied electronics during Phase II (Harold Hughes, NRL, as noted in Key Personnel above). However, NRL personnel would be available for designing the radiation shielding of operators, if this is found to be a significant problem. NRL personnel are available for development of light and portable RF electronics, if this problem proves difficult.

Southern Cross will provide the services of Kenneth Purser, as noted in Key Personnel above.

USNA Labor with 33% overhead included

