Calculations to estimate the dose effects from the use of Varian 1800 (e-Zapper) in open field tests at Technical Options, Newark, Ohio.

References:

1.  NCRP Report No. 51, “Radiation Protection Design Guidelines for 0.1 – 100 MeV Particle Accelertor Facilities,” March 1, 1977.

2.  Meeting at Technical Options with Steve Andrews, James Ziegler, Martin Nelson – December 21, 2004.

3.  Meeting at Technical Options with Steve Andrews, James Ziegler, Martin Nelson – February 1, 2005.

Experimental Setup:

Soil Bunker

Output Port

e-Zapper

Primary Beam (20m)

# Target

Dark Current

# Control

Cable

(13m)

# Aluminum

Beam Stop

Exclusion

Boundary

Soil Bunker

# Operator

Location

Assumptions / Conditions:

(1) Electron beam energy at output port = 9 MeV 3.

(2) Accelerator operates at 275 mAmps, with ½ % duty cycle 3.

(3) Operators at approximately 50 feet, which has been rounded up to 13 meters 3.

(4) Electrons leave accelerator through 2 mm output port and spread at an angle of 15o 2,3.

Beam

Output

Port (d=2mm)

15o

(5) Target Located 20m from output port

(6) Beam stop constructed of aluminum.

(7) Neglect interactions of electrons with surrounding concrete pipe when analyzing beam stop.

(8) Dark current emitted behind accelerator has magnitude of 350 mrem/hr at 1m from system  and will always be in existence even when accelerating voltage is not applied to tube 2.

(9) All equations, calculations, interpolations of graphs – based on NCRP report #51 1

(10) Quality Factor (Q) is unity for both x-rays and electrons.

(11) Neglect loss of electrons in air when hitting target (assumption recommended to be made by NCRP 51).

(12) Target can be treated as low Z material.

## Electron Projected Range and Range in Air/Aluminum/Target

Rpr = 0.53E – 0.106						(App B.2)1

where

Rpr = projected electron range, g/cm2

E = electron energy , MeV

| ### Medium | Medium Density () (g/cm3) | Rpr (g/cm2) | Range (Rpr/) (cm) | (Rpr/) (inches) |
| --- | --- | --- | --- | --- |
| Air | .0012 | 4.66 | 3883 | 1529” or 38.8m |
| Aluminum | 2.7 | 4.66 | 1.73 | .68 |
| Plastic | 1.0 | 4.66 | 4.66 | 1.83 |

Beam Cone Characteristics – Radius, Area at 20m from Output Target with 15o Angle of Spread

Beam

Output

Port (d=2mm)

7.5o

7.5o

20m

Cone at Target

tan 7.5o = x/20m

X = 2.63m

X = radius of cone at 20m

Cone Area = R2 = (2.63m)2 = 21.7m2

Cone Area = 21.7 x 104 cm2 = 2.17 x 105 cm2

(at 20m)

(at 20m)

Average Absorbed Dose Index Rate at Target (D)

D = EIRpr x 108  radssec	(App. B.2)1

where

| #### E | = | Electron Energy, MeV |
| --- | --- | --- |
| I | = | Current Density of electrons impinging on surface of material, mA/cm2 |
| Rpr | = | Projected Electron Range, g/cm2 |

For system being tested:

Current = 275 mAmps

Output Port aperture diameter = 2mm = 0.2cm

Output Area - R2 = (0.1cm)2 = 0.0314cm2

Duty Cycle = 0.5% = .005

I = 275mAmps0.0313cm2 x .005 = 43.8 mAmpscm2

Rpr (air) = 3883 cm

E = 9 MeV

At Output Port:

D = (9)(43.8)4.66 x 108 = 8.46 x 109 radssec

At Target (20m from Output Port):

I = 275mAmps2.17x105cm2 x .005 = 6.33 x 10-6 mAmpscm2

D = (9)(6.33x10−6)4.66 x 108 = 1.22 x 103 radssec

X-ray Emission Rates in Beam Stop:

For 9 MeV incident electron:

Beam Stop

Angular Emission Factor

DI=5 x 104 rads⋅m2mAmps⋅min x 	E.11

where:

Beam Stop

Angular Emission Factor

= 0.5  (at 0o in Aluminum/Concrete)	E.31

In sideward direction (90º)

DI=1 x 103 rads⋅m2mAmps⋅min                                                           E.11

I = Beam intensity at beam stop, mAmpsm2

0o Angle, 20 m from output port:

Assume beam stop at same distance from output port as target (20m)

I = 6.33x10-6 mAmpscm2 x 104 cm2m2 = 6.33 x 10-2 mAmpsm2

D= 6.33 x 10-2 mAmpsm2 x 5 x 104 rads⋅m2mAmps⋅min x 0.5

D= 1.58 x 103 radsmin

90o Angle, 20m from output target:

D90o =  6.33 x 10-2 mAmpsm2 x 1 x 103 rads⋅m2mAmps⋅min   =6.33 x 101 radsmin

X-ray emission rate (Q=1):

| Angle of emission | radssec | remsec |
| --- | --- | --- |
| 0o | 26.3 | 26.3 |
| 90o | 1.05 | 1.05 |

Shielding requirement to reduce dose rate to 2 mrem/hr limit at exclusion boundary of tests.

Forward

Direction

Primary Beam

Aluminum

Beam Stop

90o

0o

Assumptions:

(1) Each shot is of 1-second duration

(2) 5 shots are performed each hour

(3) Exclusion boundary at beam stop

At beam stop, dose rate would be:

| Angle of emission | remhr |
| --- | --- |
| 0o | 26.3 remshot x 5 shothour = 131.5 remhr |
| 90o | 1.05 remshot x 5 shothour = 5.25 remhr |

Note:  (Doses are interpolated as being at 1m from beam stop per ref 1).

10th Value Layer Thickness of Soil

10th  Value Layer, Thickness of concrete for 9MeV electrons

=  15 inches 	Fig E.121

NCRP says that one should adjust concrete’s 10th value thickness by ratio of density of comparison material to that of concrete.

Assume:  ρsoilρconcrete = 0.833

TVLsoil = 10th value layer thickness of soil for 9 MeV electrons

= (1.2)(15 in.) = 18 inches

Soil requirement (t) at 0o angle to beam:

2 mremhr = 131.5 x 103 mremhr x 10(-t/TVLsoil)

21.31x105 = 10(-t/TVLsoil)

65500 = 10(-t/TVLsoil)

log10 (65,500) = t/TVLsoil

t = TVLsoil log1065,500

t = 18 in. (4.82) = 86.8 inches

Soil requirement at 90o angle to beam:

2 mremhr = 5.25 x 103 mremhr x 10(-t/TVLsoil)

t = TVLsoil log10(2625) = 61.5 inches

## Shield Design

Primary Beam (20 m)

Target

Aluminum

Beam Stop

61.5 in.

86.8 in.

Soil Shield

Dose to Operators:

Assume beam contained by a 1-meter diameter concrete pipe as shown below:

e-Zapper

Control

Cable

(13m)

Operator

Location

Output Port

Primary Beam (20m)

Target

Aluminum

Beam Stop

Target Box

Beam Access Ports

Concrete Pipe

do = 23.8

Soil Barrier

do =

Ground Level

X-rays emitted at 135o angle of incidence.  Assume x-ray emission rate same as that at 90o angle or 5.25 Rem/Hr (this is dose at a distance of 1m from surface of beam stop as per explanation in NCRP #51).

This would be attenuated by 1r2 law from target/beam stop to the operator location.

Dop=5.25Remhr1do2=5.25123.8

Dop=.0093Remhr=9.3mRemhr

Soil requirements to operators:

2 mremhr=9.3mremhrx10−tTVLsoil

t = TVLsoil log 4.65

t = 18 inches (.667) = 12.0”

Effect of Concrete Pipe on X-Ray production:

- Assume as done in operator dose calculation, that the beam is surrounded by a 1m concrete pipe

e-Zapper

Output Port

Primary Beam (20m)

Target

Aluminum

Beam Stop

Target Box

Beam Access Ports

Concrete Pipe

X

7.5o

Beam Divergence

Intersection

= X-Ray Production

X-Ray Production will occur when beam diverges sufficiently such that it intersects the concrete pipe.  Determine X:

X

Beam

Output

Port (d=2mm)

7.5o

7.5o

Cone at  Pipe Intersection

½ m

tan 7.5o = 12mX

X = 12mtan7.5 = 3.8 m

Intersection occurs at 3.8m from output port.

Assume intersection occurs linearly between 4.0m and target.

Cross-sectional Area of concrete pipe (1/2 m diameter) = .785 m2.

Cross-sectional of beam if unrestricted and grows at

= 21.7 m2  (See page 2)

7.5o half angle from centerline axis to target/beam stop

at 20m.

= 21.7−.78521.7 = 96%

Fraction of beam which

intersects concrete

X-Ray emission rates (9 MeV electrons)

D=I⋅5x104m2mampmin⋅(Concrete Angular Emission Factor)	(E.1)

At 0o angle:

I = 275mAmpsAbeam1r2  (r = distance from output port)

Abeam = Beam cross-sectional area, cm2

I will decrease as 1r2 of the distance from the output port.  However, the distance the x-rays emitted in the x-direction will increase by r2 to the beam stop.  Hence there is no change in the x-ray production in the 0o angle and the same thickness of soil is required to reduce the dose to 200 mrem/hr at the boundary of the exclusion zone.

At 90o emission angle (neglect 7.5o intersection angle):

DI=   1 x 103 rads⋅m2mAmps⋅min                                                        E.31

Since 96% of beam intersects with concrete between 4m & 20m

IntersectionMeterRate = 96%meter = 6%m

The x-ray production rate will vary directly with electron beam intensity striking the concrete.  The electron beam intensity will vary inversely proportional to the beam’s cross-sectional area, which in turn will vary as r2 from beam output port or increase as  (20+rbs20)2    from beam stop.

90o x-ray production at distance rbs from beam stop

(5.25Remhr)(20+rbs20)2

=

rbs = distance from beam stop, m

5.25Remhr = X-Ray production in beam stop at 90o

Assume concrete ρ = 1.2 grams/cm3

Range of electrons in concrete = 4.661.2 = 3.9 cm  (1.52”)

Note:  All electrons are stopped by 2” of concrete.

| Distance from output port (meters) | Distance from Beam Stop (meters) | 90o x-ray production (Rem/hr) |
| --- | --- | --- |
| 20 | 0 | 5.25 |
| 19 | 1 | 5.78 |
| 18 | 2 | 6.35 |
| 17 | 3 | 6.94 |
| 16 | 4 | 7.56 |
| 15 | 5 | 8.20 |
| 14 | 6 | 8.87 All dose rates at 1m from surface perpendicular to electron beam |
| 13 | 7 | 9.57 |
| 12 | 8 | 10.29 |
| 11 | 9 | 11.04 |
| 10 | 10 | 11.81 |
| 9 | 11 | 12.61 |
| 8 | 12 | 13.44 |
| 7 | 13 | 14.28 |
| 6 | 14 | 15.17 |
| 5 | 15 | 16.07 |
| 4 (First intersection of beam with concrete) | 16 | 17.01 |

Average X-Ray Production = 10.61Rem/hr at a distance of 1m from comcrete.

Note:  Because of the high x-ray production, no personnel will be allowed next to the concrete pipe when the system is in operation!!

X-Ray Production in lateral direction in concrete:

Soil Barrier

20m

Output Port

7.5o

e-Zapper

Primary Beam (20m)

# Target

# Aluminum

Beam Stop

Concrete Pipe

4m

No x-ray

Production

Beam Divergence

Intersection

###### Target Box

= X-Ray Production

Assume each x-ray component must travel 20m to reach exclusion boundary.  Soil barrier can be located anywhere inside exclusion area.  Location based on ease of facility operation.

DLateral = (10.61 Rem/hr) (1m/20m)2

D= 0.0265 Rem/hr = 26.5 mrem/hr

Determine thickness of soil needed to reduce dose to 2 mrem/hr:

2 mremhr=26.5mremhrx10−tTVLsoil

t = TVLsoil log10 (13.25)

t = 18 inches (1.12) = 20.2”

e-Zapper

Output Port

Primary Beam (20m)

Target

Aluminum

Beam Stop

Target Box

Concrete Pipe

4m

No x-ray

Production

7.5o

Beam Divergence

Intersection

= X-Ray Production

Soil Barrier

20.2”

Soil Barrier

20.2”

Extending the soil barrier entirely along length of concrete pipe will then provide needed protection for operators.  Operators to be moved back 20m.

Dark Current Shield

- Exists at 180o from e-Zapper beam port

- May exist even when machine voltage is not applied

350 mrem/hr

e-Zapper

rdc

rdc  = distance required to reduce dark current (dc) to 2 mrem/hr from dark current, m

Ddc=(350mremhr)(1rdc)2=2mremhr

rdc2 = 175 m2

rdc2 = 13.2 m   Note:  Distance to exclusion area must be greater than or equal to this value.

Reflection / Scattering of x-rays from soil to operators

Soil Barrier

20.2”

Output Port

7.5o

e-Zapper

Primary Beam (20m)

# Target

# Aluminum

Beam Stop

Concrete Pipe

4m

Beam Divergence

Intersection

###### Target Box

13 m

Soil Barrier

20.2”

Assume dose rate (from exit port) reflected, that at 4m (256 rem/hr):

Dose rate that reached soil barrier (4m from exit port)

Attenuation in soil 20.2”

Distance Attenuation

Dref = (		) α (		     )  ( 		     )

α = Albedo for 9 Mev x-rays off concrete, (assumed concrete has same value).

α = 3 x 10-3, figure E-151

Assume rref = distance x-rays must be reflected to reach operators, taken to be 13 m

D = (17.01 x103 mrem/hr) (3x10-3)  (10-(20.2/18)) (1m/13m)2

D = .02 mrem/hr

Potential Shield Design Around e-Zapper with exclusion zone:

e-Zapper

Output Port

Primary Beam (20m)

Target

Concrete Pipe

4m

No x-ray

Production

Beam Divergence

Intersection

Soil Barrier

20.2”

Soil Barrier

20.2”

Soil

Barrier

61.5”

Soil

Barrier

61.5”

Soil

Barrier

86.8”

EXCLUSION AREA

EXCLUSION AREA

(2 mrem/hr)

(2 mrem/hr)

(2 mrem/hr)

(2 mrem/hr)

OPERATOR STATION

20m

20m

13.2m

2

mrem/hr

`

Soil requirement behind beam stop and on its sides can be reduces by expanding exclusion area.